NATIONAL UNIVERSITY OF                COMPUTER & EMERGING         SCIENCES

PROJECT:
                    DIGITAL LOGIC DESIGN

TOPIC:

4 BIT COUNTER D FLIP FLOP

GROUP MEMBERS:

ASJID TAHIR (19p-0085)

IBRAR BABAR (19p-0104)

AQEEL ABBAS (19p-0100)

Working Software:
   					We can make our circuit in soft form on software (Logisim).

Link for Download Logisim Software :

https://sourceforge.net/projects/circuit/

Definition:

A 4-bit synchronous counter built from D-flipflops with carry-input and carry-output. In this circuit, the single clock signal is directly connected to all flipflops, so that all flipflops change state at the same time.

D-Flip Flop:

Table:

| Pin | Input/Output | Description |
| --- | --- | --- |
| D | Input | Data input |
| CLK | Input | Clock input |
| Q<3:0 | Output(4-bits) | Count Output |

Explanation:

A 4-bit synchronous counter built from D-flipflops with carry-input (count-enable) and carry-output. In this circuit, the single clock signal is directly connected to all flipflops, so that all flipflops change state at the same time. It consists of:

- 4 Ex-OR gates.

- 4 OR gates.

- 6 AND gates.

- 1 NOT gate.

- 1 Clock.

- Set/Reset State.

- Up/down.

In this circuit, the input of D-flipflop is connected with the clock and data pin of D-filpflop is connected with the output of the ex-or gate and the bottom part of D-filpflop which is actually known as “clear” is connected with the reset button.The output part of D-flipflop is connected with four pins from which we can observed that our 4 bit counter is in set state or reset state. The output of or gates is connected with the second input of ex or gate. The first input of ex-or gate is connected with the second input of second and gate. Not gate is short circuit by up and down button. The output of two and gates is connected with input of or gate.

Excitation table for D-Flipflop:

| Present State (Q) | Input (D) | Next State (Q+) |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

State Diagram of bit counter:

Truth Table of 4 bit counter:

| Present State (Q3 Q2 Q1 Q0) | Next State (Q3+ Q2+ Q1+ Q0+) | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- |
| 0000 | 0001 | 0 | 0 | 0 | 1 |
| 0001 | 0010 | 0 | 0 | 1 | 0 |
| 0010 | 0011 | 0 | 0 | 1 | 1 |
| 0011 | 0100 | 0 | 1 | 0 | 0 |
| 0100 | 0101 | 0 | 1 | 0 | 1 |
| 0101 | 0110 | 0 | 1 | 1 | 0 |
| 0110 | 0111 | 0 | 1 | 1 | 1 |
| 0111 | 1000 | 1 | 0 | 0 | 0 |
| 1000 | 1001 | 1 | 0 | 0 | 1 |
| 1001 | 1010 | 1 | 0 | 1 | 0 |
| 1010 | 1011 | 1 | 0 | 1 | 1 |
| 1011 | 1100 | 1 | 1 | 0 | 0 |
| 1100 | 1101 | 1 | 1 | 0 | 1 |
| 1101 | 1110 | 1 | 1 | 1 | 0 |
| 1110 | 1111 | 1 | 1 | 1 | 1 |
| 1111 | 0000 | 0 | 0 | 0 | 0 |

K –Map:

Now we use the k-map to find out the logic expressions D0, D1, D2 and D3.

So, we found the value of D3, D2, D1, D0 in terms of Q3, Q2, Q1.

Circuit Diagram:

Following is the paper work of the circuit:

Circuit Diagram: